Koolish Zein  - BalletCollective Aotearoa & Naoto Segawa

# Production Requirements

Staging:

Black Dance floor

Standard masking

Smother on flyline 54

Cyclorama flyline 57 (if available)

Carpet runners side of stage

Hazer

4 x music stands

Stage manager to call from side of stage with monitors.

Costumes – 1 extra suitcase with cast.

Sound:

A PA that will cover every seat in the house with a Digico SD11 or Midas Pro 1 console with the appropriate I/O. Note here that we will require subs as it’s not just the Trio but also backing tracks with potential low frequency content.

A redundant Qlab play back system, the Naoto will need click track for some of the pieces.

Monitors from both stage left and stage right either flown or on a stand (this may need to be multiple positions depending on how deep the stage is, I will take your advice for this).

Something like a d&b audiotechnik E8 is perfect for this kind of thing and would be easily flown or mounted.

4 x DPA 4099 microphones with 1 x violin mount, 1 x cello mount, 2 x piano mount

3 x stereo in ear monitor systems with 3 packs and appropriate antenna. Shure PSM1000 or Sennhesier G3/4.

Cabling to suit.

Lighting:

House rig 3 colour wash.   4 booms per side with 4x Source 4 26 degree profile & 1 PC 1200

1 Par 64 cp61 unit on each side ladder.

12x profile units FOH Bridge #2

Curtain warmers from FOH.

Cyc lights

Projector

ETC ION lighting desk

Crew:

Venue Manager

Venue Tech/Flyman

Sound/AV Op

Lighting Op